For 8086 Assignment 1: Add two 8-bit numbers

MOV AX, 2000H

MOV DS, AX

MOV SI, 5000H

MOV AL, [SI]

MOV BL, [SI+1]

ADD AL, BL

MOV [SI+2], AL

HLT

Add two 16-bit numbers,

MOV AX, 2000H

MOV DS, AX

MOV SI, 5000H

MOV AX, [SI]

MOV BX, [SI+2]

ADD AX, BX

MOV [SI+4], AX

HLT

Subtract two 8-bit numbers,

MOV AX, 3000H

MOV DS, AX

MOV SI, 9000H

MOV AL, [SI]

MOV BL, [SI+1]

SUB AL, BL

MOV [SI+2], AL

HLT

Multiply two 8-bit numbers

MOV AX, 4000H

MOV DS, AX

MOV SI, 3000H

MOV AL, [SI]

MOV BL, [SI+1]

MUL BL

MOV [SI+2], AL

MOV [SI+3], AH

HLT

Divide two 16-bit numbers

MOV AX, 5000H

MOV DS, AX

MOV SI, 3000H

MOV AX, [SI]

MOV DX, 0000H

MOV BX, [SI+2]

DIV BX

MOV [SI+4], AX

MOV [SI+6], DX

HLT

A two-digit BCD number is stored in memory location 51000H. Unpack the BCD number and store the two digits in memory locations 52000H and 52001H such that memory location 52000H will have the lower BCD digit and 52001H will have the upper BCD digit.

MOV AX, 5000H

MOV DS, AX

MOV SI, 2000H

MOV BL, [1000H]

AND BL, 0FH

MOV [SI], BL

MOV BL, [1000H]

AND BL, 0F0H

ROR BL, 04H

MOV [SI+01H], BL

HLT

A two-digit BCD number is stored in memory locations 31000H and 31001H. Pack the BCD number and store the packed BCD number in memory location 32000H.

MOV AX, 3000H

MOV DS, AX

MOV SI, 1000H

MOV DI, 2000H

MOV BL, [SI]

ROL BL, 04H

MOV AL, [SI+01H]

OR BL, AL

MOV [DI], BL

HLT

Write a program in 8086 microprocessor to find out the largest among 8-bit 10 numbers. The numbers are stored from memory address 3000:5000, and store the result (largest number) into memory address 3000:6000.

MOV AX, 3000H

MOV DS, AX

MOV SI, 5000H

MOV CL, 0AH

MOV AL, [SI]

MOV BL, AL

INC SI

DEC CL

J2:

MOV AL, [SI]

CMP BL, AL

JNC J1

MOV BL, AL

J1:

INC SI

DEC CL

JNZ J2

MOV [6000H], BL

HLT

Write an assembly language program in 8086 microprocessor to check whether the last two digits of the university roll number stored at memory location 52000H is even or odd. If it is even, store 00H at memory location 52001H; otherwise, store FFH at memory location 52001H.

MOV AX, 5000H

MOV DS, AX

MOV SI, 2000H

MOV BYTE PTR [SI], 06H

MOV AL, [SI]

AND AL, 01H

JZ EVEN

MOV BYTE PTR [SI+01H], 0FFH

JMP STOP

EVEN:

MOV BYTE PTR [SI+01H], 00H

STOP:

HLT

Write a program to transfer a block of 5 bytes, starting address is 5000H and transfer the block at address 6000H by using string instructions. DS = 4000H.

MOV AX, 4000H

MOV DS, AX

MOV AX, 5000H

MOV ES, AX

MOV SI, 5000H

MOV DI, 6000H

MOV CX, 05H

CLD

REP MOVSB

HLT

Write and execute an ALP to 8086 Microprocessor to find the length of a given string which terminates with a special character.

MOV AX, 3000H

MOV DS, AX

MOV SI, 4000H

MOV BL, 24H

MOV CL, 00H

J2:

MOV AL, [SI]

CMP AL, BL

JZ J1

INC CL

INC SI

JMP J2

J1:

MOV [5000H], CL

HLT

Write an assembly language program for 8086 to reverse a string.

MOV AX, 3000H

MOV DS, AX

MOV CX, 05H

MOV SI, 4004H

MOV DI, 5000H

L1:

MOV AL, [SI]

MOV [DI], AL

DEC SI

INC DI

DEC CX

JNZ L1

HLT

Write an assembly language program for calculating the factorial of a number using 8086 microprocessor.

MOV AX, 5000H

MOV DS, AX

MOV SI, 8000H

MOV BL, [SI]

MOV AL, 01H

MOV AH, 00H

CMP BL, 00H

JZ END_FACT

L1:

MUL BL

DEC BL

JNZ L1

END_FACT:

MOV [8002H], AX

HLT

Write an assembly language program in 8086 microprocessor to find the square root of a number.

MOV AX, 3000H

MOV DS, AX

MOV SI, 4000H

MOV AL, [SI]

MOV BL, 01H

MOV CL, 00H

L1:

CMP AL, BL

JB DONE

SUB AL, BL

INC CL

ADD BL, 02H

JMP L1

DONE:

MOV [4002H], CL

HLT

Write an ALP program to transfer a data from memory to output port.
Port No. – 112 (00C7H), Data – Last 4 digits of University Roll No., Memory Address – 35000H

MOV AX, 3000H

MOV DS, AX

MOV SI, 5000H

MOV AL, [SI]

MOV DX, 00C7H

OUT DX, AL

HLT

Write an ALP program to transfer data from an input port to a memory address.
Port No. – 110 (006EH), Data – Last 2 digits of University Roll No., Memory Address – 62000H

MOV AX, 6000H

MOV DS, AX

MOV DX, 006EH

IN AL, DX

MOV [2000H], AL

HLT

Write an ALP program to interface LED Display.

MOV AX, 3000H

MOV DS, AX

MOV AL, 0FFH

MOV DX, 00C7H

OUT DX, AL

HLT

Write a program to interface a traffic light controller.

J3:

MOV AX, 514H

OUT 4, AX

CALL DELAY

MOV AX, 861H

OUT 4, AX

CALL DELAY

MOV AX, 8A2H

OUT 4, AX

CALL DELAY

MOV AX, 30CH

OUT 4, AX

CALL DELAY

JMP J3

DELAY:

MOV CX, 0058H

J2:

DEC CX

JNZ J2

RET

Write an ALP program to rotate a stepper motor in clockwise direction or anticlockwise direction.

MOV DX, 7

L1:

MOV AL, 09H

OUT DX, AL

MOV AL, 0CH

OUT DX, AL

MOV AL, 06H

OUT DX, AL

MOV AL, 03H

OUT DX, AL

JMP L1

8051

a 16-bit addition using two 8-bit additions (low byte + high byte with carry)

ORG 8000H

MOV A, #26H      ; First number

MOV B, #62H      ; Second number

ADD A, B         ; A = 26H + 62H

MOV 8502H, A     ; Store result at 8502H

END

Write a program for addition and subtraction of two 16-bit numbers using Edsim51 software, you can write it in the same style as your previous assignment.

16-bit Addition (8051)

ORG 8000H

MOV A,#32H

MOV B,#62H

ADD A,B

MOV R0,A

MOV A,#14H

MOV B,#43H

ADDC A,B

MOV R1,A

END

16-bit Subtraction (8051)

ORG 8000H

MOV A,#62H

MOV B,#32H

CLR C

SUBB A,B

MOV R0,A

MOV A,#43H

MOV B,#14H

SUBB A,B

MOV R1,A

END

Write an assembly language program for multiplying and dividing two 8-bit numbers using EdSim51, write it like this.

- 8-bit Multiplication (8051)

ORG 8000H

MOV A,#14H

MOV B,#05H

MUL AB

MOV R0,A      ; Lower byte of result

MOV R1,B      ; Higher byte of result

END

- 8-bit Division (8051)

ORG 8000H

MOV A,#64H

MOV B,#05H

DIV AB

MOV R0,A      ; Quotient

MOV R1,B      ; Remainder

END